INVITATION TO BID (ITB) AND TERMS OF REFERENCE (ToR) FOR CONSTRUCTION IRRIGATION INFRASTRUCTURE IN GIDHEY VILLAGE, SHEIKH.

About ADO

The Agricultural Development Organization - ADO - was established in 1993 and had worked throughout the Somaliland regions, implementing emergency, recovery, and long-term rural development projects, including agriculture, environment, and climate change adaptation, resilient livelihood building for the poor agro-pastoralists, watershed, and irrigation management, nutrition, community innovations, and entrepreneurship support. Besides, the ADO had empowered rural women through the needs-based small business projects and Self-Help-Group initiatives. Working directly with community associations, the ADO had supported more than 776,000 households, including IDPs, returnees, farmers, and women living throughout the Somaliland regions.

The ADO has its Head Office in Hargeisa with the regional branch offices in Burao, Borama, and Erigavo of Togdheer, Awdal, and Sanaag regions. Besides, the ADO is currently expanding its programs and presence in the Somali Ethiopia Region, South-central, and Southern Somalia. ADO is an active member of the main local umbrella groups, namely, NAGAAD, SOLPAF, and SONSEF. As well as the ADO has strong networks with academic institutions in Europe and Africa and collaborates with diverse institutions in the development and agriculture research sectors. The partners of the ADO include but are not limited to the European Union, UNHCR, UNDP, WFP, UNEP, Oxfam, Care International, ISF Finland, NDF Norway, NORAD, UK AID, WHH Germany, Somaliland Development Fund (SDF), KNH Germany, and LUKE of Natural Resource Institute of Finland.

The project named “Women Empowerment for Climate and Economic Resilience (WECARE)” is a four-year (2026-2029) initiative that will be implemented by the Agricultural Development Organization (ADO) with support from the International Solidarity Foundation (ISF). The project is designed to enhance the resilience of agropastoral communities in Somaliland through an integrated, multidisciplinary approach. The project addresses the interconnected challenges of climate change, food insecurity, gender inequality, and disaster vulnerability within a cohesive and inclusive framework.

WECARE will be implemented in the villages of Gidheys and Kaladhac villages, located in Sheikh and Burao districts of the Sahil and Togdheer regions—areas recognized as among the most socio-economically marginalized and environmentally fragile in Somaliland. Despite their rich cultural heritage, these rural and semi-nomadic communities remain underserved by humanitarian and development actors and face persistent cycles of poverty, climate-related shocks, and gender-based exclusion.

The project will directly target 750 households (450 female-headed and 300 male-headed) and reach 4200 final beneficiaries, including children and adults from farming and pastoralist communities, farmer cooperatives, self-help groups (SHGs), livestock traders, and other value chain actors. 60% of total direct beneficiaries will be targeted vulnerable women, and five percent (5%) of the direct beneficiaries will be persons with disabilities, ensuring inclusive participation in the project.

# 1.1. Description of programme location

The ISF project aims to provide access to improved accessibility of water for irrigation purpose, addressing the challenges caused raised fuel used for generators and leaded farming production reduced Gidheys village. In addition, supporting water infrastructure will be developed, including pipeline extensions, construction of elevated water tanks, water kiosks farms, installation of a solar-powered system, and supply additional HDPE pipes. Consequently, ADO extended an invitation to local bidders in Somaliland.

- GENERAL INFORMATION ON THE INVITATION TO BID (ITB).

All interested and eligible suppliers/contractors with proven capacity and relevant experience in similar work are invited to submit their bids.

2.1. Currency

All bid prices must be quoted and submitted in (USD) currency.

# 2.2. Modification and withdrawal of bids

A bid may be withdrawn upon written request submitted prior to the bid closing date. Any amendments or revisions to the bid must also be submitted before the closing date.

2.3. Signature of the contract.

ADO in Somaliland will inform the successful bidder through email and will send the contract form within 3 weeks after closure of the submission deadline. The successful bidder shall sign and date the contract and return to ADO in Somaliland within seven calendar days of receipt of the contract. After the contract is signed by two parties, the successful bidder shall deliver the services in accordance with the delivery of BOQ, and design outlined in the bid.

2.4. Rights of ADOs:

- Contact any or all references supplied by the bidder(s).

- Request additional support or supplementary information from the bidder(s).

- If any clarification is needed for the successful bidder, ADO will communicate through emails or verbally.

- Negotiate with the bidder who has attained the best rating/ranking, i.e. the one(s) providing the best overall value.

2.5. Data Confidentiality and Data Transfer Requirements.

The Contractor shall maintain strict confidentiality of all information, observations, and data collected during the construction works. No data or findings may be shared with third parties without written authorization from ADO.

Failure to comply with these requirements may result in suspension of work and/or contractual penalties.

TERMS OF REFERENCE (TOR)

1. SCOPEOF WORK:

The contractor is required to construct Irrigation water infrastructure in Sheikh District, Sahil Region, Somaliland. The contractor shall construct and install all required infrastructure and pipeline extensions to a high standard and in accordance with the approved designs, technical specifications, and Bill of Quantities (BoQ).

# 2. The main activities to be undertaken

- Installation of equipment of submersible pump and Cables as specified in the BOQ.

- Installation of a solar-powered system as specified in the BOQ.

- Construction of a new reinforced cement concrete (RCC) elevated water tank with a capacity of 10m³ and a height of 3-6 meters as specified in the BOQ.

- The water kiosks and installation & connection of the existing ground water tank to the pipeline network for use as a water kiosk as specified in the BOQ

- Pipeline extension as specified in the BOQ.

- Additional HDPE pipe supply for irrigation purpose

## 2.1. Installation equipment submersible pump and Cables

- Supply and install 3-inch uPVC column riser pipes (5.8 meters per length), including all necessary fittings such as sockets, adapters, and other required accessories.

- Supply and install a multistage 1.5 kW submersible pump (Grundfos or Lowara). The pump shall have a minimum head of 70-100 m and a minimum discharge capacity of 3-5 m³/hour against a pump head of 300 m.

- Supply and install 3-core 10 mm² pure copper pump cable.

- Supply and installation of 2-inch HDPE pipes (100m length each roll), including sockets and fittings.

N.B: Only technically experienced personnel shall carry out the commissioning and installation of the submersible pump, and cable inside the spring.

The contractor shall be fully responsible for any damage caused to the spring during the commissioning, installation of the pipes, submersible pump, and associated cabling works.

## 2.2. Installation of a solar-powered system

- Monocrystalline solar panels with a Total peak power min 3.6kw: Panel power rating 600W in STC, Voltage max 24.6V, Current max 15.15A, WITH 132 Cells, efficiency>20, 240V system voltage, with manufacturer’s datasheet.

- Connector Type MC4.

- Warranty of minimum 10 years

- Note: The supplier may use larger/smaller solar panels with the same amount of power required but he/she must inform the ADO engineers for approval.

- Supply and installation of INVT/RSI solar pump inverter, 3-phase 380-440V, IP66 weatherproof protection, 3.6 kW, 38A, suitable for operation and protection of 3.6KW submersible spring pump, complete with dry-run protection, overload protection, over/under voltage protection, phase failure protection, automatic restart function, and hybrid operation compatibility with solar power and generator systems

- Provide a complete solar protection cabinet, including:

  - PV Combiner Box

  - PV Disconnect Switch

  - Surge Protection Device (SPD)

  - Highly sensitive AC and DC automatic circuit breakers/MCBs (Miniature Circuit Breakers)

- Solar Mounting Structure: Supply and install 3-inch Class-B GI pipes for the columns of the ground-mounted structure, with concrete foundations at the base. Provide all required steel grids/rafters using minimum 8 cm × 4 cm pre-painted steel box sections and 6 cm × 3 cm steel purlin box sections, in accordance with the approved design drawings.

## 2.4. Construction of New RCC Elevated Water Tank

Construct, test, and commission a new RCC elevated water tank with a capacity of 10 m³ and a height of 6 m above ground level near Gidhey Site. The works shall include site preparation, excavation, RCC foundations, columns, beams, staircases, tank structure, waterproofing, inlet/outlet/overflow/drainpipe, access ladder, plastering, painting, drainage works, testing, and commissioning in accordance with approved drawings, BOQs and specifications.

## 2.5. Construction of Water Kiosk for farm

Construct and complete water kiosks with associated within Gidhey Togga Siir Site. The works shall include excavation, concrete foundations, masonry/block works, plastering, water tap stands, drainage systems, paving works, and installation of chain-link or GI sheet fencing with gates, including all necessary fittings and accessories, in accordance with the approved drawings, BOQ, and technical specifications. The work shall also include installation and connection of the existing ground water tank to the pipeline network for use as a water kiosk.

## 2.6. Pipeline Extension Works

Supply and install pipeline extensions including pipes, fittings, excavation, backfilling, testing, and commissioning. The pipes should be reflected in the specifications mentioned in the Bill of Quantities.

## 2.7. 	Construction of Fencing Works

Construct site fencing including fence posts, chain-link concrete foundations, gate installation, and all related accessories as specified in the BoQ.

3. Specifications of construction materials and mixing ratio:

## 3.1. Earth work

Site clearance: Site clearance will be included removing of all rubbish,derbis, trees and unnecessary material arising in approved locations. Contractor shall be responsible for the disposal of surplus excavated materials off site if any.

3.2. Water: The water used for mixing concrete and mortar shall be clean, fresh, and free from harmful substances. The water should be of potable quality and suitable for drinking.

3.3. Sand: sand used for masonry works and mixing concrete shall be clean and good quality it should be free from salt, organic matter and other deleterious substances. Preferably, sand from the river shall be used for the work.

3.4. Stone: Stone used for masonry work must be taken from the mountain and does not form the river and the stones should be suitable for the work as directed by the ADO Engineer.

3.5. Cement: Omani Cement with 42.5n/mm2 will be mandatory use, any other cement is not allowed to be transported to the site.  Please note that cement which is not fresh and dry before mixing shall not be used in the work.

3.6. Aggregate: Fine aggregate shall be clean natural sand. Coarse aggregate shall be crushed stone, washed gravel Aggregate used will be 10mm to 35mm size.

3.7. Steel reinforcement:

The steel reinforcement will be mild steel with a minimum of 355n/mm2 tensile strength, size of bars will be 8mm for stirrups, 16mm for footings and columns, beams and 12mm bars for slabs and elevated tank walls. Please follow the details on the bill of quantities and design.

3.8. Mixing ratio: for all RCC works the mixing ratio shall be 1:1.5:3 for cement, sand and gravel for all clean concrete works the mixing ratio shall be 1:2:4 for cement, sand and gravel for mortar and plastering the mixing ration shall be 1:3 cement and sand respectively.

# 4. Mobilization and Demobilization

Mobilization shall include the provision and transportation of all necessary manpower, equipment, tools, and construction materials to the project site. The Contractor shall be fully responsible for providing all personnel, equipment, and materials required for the successful execution and completion of the contracted works. The Client shall not be held responsible for any delays or additional costs incurred due to the Contractor’s failure to mobilize adequate or sufficient equipment for the works specified under the contract.

Demobilization shall include all clean-up work and related operations necessary for the removal of personnel, equipment, temporary facilities, waste materials, and all other incidentals from the project site upon completion of the works.

- Environmental Safeguarding:

The Contractor shall take all necessary environmental protection and safeguarding measures throughout the execution of the works to prevent or minimize adverse environmental impacts. This shall include, but not be limited to, proper waste management, safe handling and disposal of waste materials, control of pollution, protection of surrounding areas, and compliance with all applicable environmental laws, regulations, and approved environmental standards.

- ADO Engineer:

The ADO Engineer will oversee the implementation of the work in accordance with the contract, BOQ, design and specifications. The ADO Engineer is responsible for all technical matters related to the contractor’s work. The responsibilities of the ADO Engineer include, but are not limited to, the following:

  - Communicate with the contractor, in writing, about the selected construction sites.

  - Provide written instructions for commencement of the work and issue certificates of provisional and final completion.

  - Give written approval of the proposed work plan and receive and review all other related documentation required under the contract.

  - Note: The contractor may carry out technical tasks only under the supervision and approval of the ADO Engineer. Therefore, the contractor is required to notify the ADO Engineer prior to commencing any activities.

- Contract Conditions and Variations:

The Contractor shall notify the ADO Engineer in writing of any proposed variation to the Contract and submit all relevant details for review and recommendation to the Client (ADO) for approval. No works related to the proposed variation shall commence unless and until written approval is issued by the Client to the Contractor. Any variation works executed without prior written approval from the Client shall be undertaken at the Contractor’s own risk, and no claim for additional payment or extension of time arising from such unauthorized variation works shall be entertained or paid by the Client.

# 9. Logistics

The contractor is responsible for all logistics required for the entire operation. This includes things such as site office, staff accommodation, etc. All consumables will have to be transported to the site at the contractor’s cost. Potable water in sufficient quantities and its transport to the worksite is the responsibility of the contractor.

# 10. Safety requirements and compliance standards during fieldwork

To ensure safe and compliant field operations throughout the construction work, the following safety measures shall be strictly enforced:

## 10.1. 	General Safety, roles and responsibility of contractor

- Assign site safety officer to regulate compliance with the safety measures.

- Installation of a fence around the working site.

- Daily safety briefings and Job Safety Analysis (JSA) shall be conducted before work begins.

- Only authorized personnel may access the worksite, with proper fencing and warning signage in place.

10.2. 	Personal Protective Equipment (PPE).

- Mandatory PPE includes safety boots, high-visibility vests, helmets, gloves, and eye/ear protection.

- Non-compliance with PPE requirements will result in removal from the site.

10.3. 	Equipment and Operational Safety.

- Only trained personnel may operate machinery and generators.

- Daily equipment inspections are required before use.

- Emergency response plan and first aid equipment should be in place.

# 11. Insurance

All required insurance for public and private liability covering injury or death and property damage or loss will be the sole responsibility of the contractor.

12. Bidding proposals and evaluations.

Separate technical and financial proposals should be submitted in sealed envelopes.

Bidders are requested to quote unit prices as per the attached BOQs. A comparison will be made based on the total prices provided, and related contracts will be formulated and issued.

- The technical proposal must be written in English.

- The technical evaluation precedes the financial evaluations. Only bidders who meet the technical evaluation score will be considered for financial evaluation.

- The bidding firm should bear in mind that the technical proposal has a weight of 60% in the selection procedure while the financial proposal has 40%.

- The sum of the technical and financial prices will determine the winner of the bid.

## 12.1. 	Preliminary evaluation criteria

| No | Requirements | Remarks |
| --- | --- | --- |
| 1 | A valid registration certificate issued by the Ministry of Public Works, Land and Housing of Somaliland. | Mandatory |
| 2 | A valid tender board license | Mandatory |
| 3 | A valid tax clearance certificate issued by the Ministry of Finance | Mandatory |
| 4 | Submit an original 1-year bank statement (June 2025–June 2026) showing a minimum balance of USD 20,000. | Mandatory |

Note: Failure by the bidder to meet this mandatory requirement will result in the bid being rejected and not considered for further evaluation.

# 13. Financial evaluation

Only bidders who meet the required technical evaluations will proceed to the financial evaluation stage. The bid price of each qualified tender will then be assessed by the evaluation committee. The sum of the technical and financial prices will determine the winner of the bid.

14. Deliverables:

Completion of the irrigation infrastructure to a high standard, in accordance with the approved BoQ, designs, and technical specifications.

# 15. Duration of contract

The Contractor shall complete all activities and works under the Contract within sixty (60 days) from the date the Contract is signed by both parties.

# 16. Payment Terms

Payments under this Contract shall be made in installments based on the progress and satisfactory completion of the Works, as verified and certified by the ADO Engineer. The payment structure shall be as follows:

First Installment – 30%:

An installment amounting to thirty percent (30%) of the total Contract value shall be paid upon completion of thirty percent (30%) of the Works, subject to verification, site mobilization, and certification by the ADO Engineer through submission of a verification report.

Second Installment – 40%:

A second installment amounting to forty percent (40%) of the total Contract value shall be paid upon full completion of the Works 70% and submission of all required documentation, including the Completion Report and any other supporting documents required under the Contract, subject to approval by the ADO Engineer.

Third Installment – 20%:

A second installment amounting to forty percent (20%) of the total Contract value shall be paid upon full completion of the Works 100% and submission of all required documentation, including the Completion Report and any other supporting documents required under the Contract, subject to approval by the ADO Engineer.

Final Installment / Retention – 10%:

The remaining five percent (10%) of the total Contract value shall be retained as retention money for a period of three (3) months commencing from the date of completion of the Works. The retained amount shall be released only after completion of the defect liability period and upon issuance of a No Defects Liability Report by the ADO Engineer. If any defects are identified during the defect liability period, the Contractor shall promptly rectify such defects to the satisfaction of the ADO Engineer prior to the release of the retained amount.

Note: Any applicable taxes shall be the responsibility of the contractor. ADO will not pay any taxes on behalf of the contractor.

Note: The documents listed below are attached separately.

- Invitation to bid (ITB) and terms of reference (ToR) for construction of Irrigation water infrastructure with Identification form

- BOQs for Construction of Irrigation water Infrastructure

- Designs for Construction of Irrigation water Infrastructure

- ADO Supplier Profile and Registration Form.

# 1. ANNEXES

## 1.1. Process for Submission of Bids and deadline

All completed tender should be summited in sealed envelope should be dropped at the tender box in ADO in Hargeisa office, a Near young Muslim Academy or Sha’ab court area area.

The titles of submitted documents should clearly state “Construction of Irrigation Water Infrastructure in Gidhey, Sheikh, Somaliland”.

Any requests for clarifications regarding this tender should be sent to yurub@adosom.org  from 5th to 16th Aug 2026 at 2:00 PM. The deadline for submitting the tender is 18/Aug/2026 at 3:00 PM.

Note: Any tender or documents received after this time will not be accepted.

## 1.2. Identification form

This bid form must be completed, signed, stamped and returned to ADO. Bids must reflect the instructions described in the bill of quantities and Terms of Reference.

The undersigned, having read the complete Request for Tender and all accompanying attachments, hereby offers to provide the specified works outlined in the BOQ and designs, in accordance with the Terms of Reference contained in this document.

Name of Bidder___________________________________________

Address, ______________________________________________

Contact Person: ________________________________________

Title: ________________________________________________

Telephone____________________________

Date:    ________________________________

Signature:_________________________           Stamp:____________________